―――――――――――――――――――――――――――――――――――――――――――――――――――――――

# FIELD ACTION GUIDE: CONFINED SPACE SUPERVISOR

Target Role: Front-Line Supervisor / Permit Issuer  |  Parent Standard: STD-CS-001

## ✔ Pre-Entry Verification Checklist

[   ]   Verify all energy sources are positively isolated per LOTO procedure (blinds/blanks/disconnects verified).

[   ]   Inspect and verify calibration dates on all 4-gas atmospheric monitors.

[   ]   Conduct pre-job toolbox talk with Entrants and Standby Person; review specific hazards and rescue plan.

[   ]   Verify rescue equipment (tripod, winch, extraction lines, SCBA) is staged at the hatch and fully functional.

[   ]   Sign and post the Confined Space Permit at the physical entry point.

## 🛑 Mandatory Stop-Work Triggers

🛑   Any atmospheric alarm activation on personal or area gas detectors.

🛑   Standby Person must leave their post without a certified replacement present.

🛑   Loss of ventilation airflow or power loss to extraction blowers.

🛑   Unanticipated chemical odors, sludge disturbance, or ingress of liquids.

⚠ LIFE SAVING RULES MANDATE Life Saving Rules Applicable: LSR-01 (Confined Space) & LSR-02 (Energy Isolation). Violations result in immediate stand-down.