―――――――――――――――――――――――――――――――――――――――――――――――――――――――

# 1. Task & Equipment Overview

| Unit / Area: | Gas Pretreatment — Warm End (AGRU) |
| --- | --- |
| Equipment Tag: | Amine Reflux Pump P-101A |
| Parent Standards: | STD-CS-001 (Confined Space), STD-EI-002 (LOTO), STD-H2S-001 (H2S Safety) |

# 2. Required Personal Protective Equipment (PPE)

Mandatory PPE: H2S Personal Monitor, Amine-Rated Splash Goggles, Face Shield, Chemical Apron, Butyl/Nitrile Gloves, Steel-Toe Boots, Hard Hat.

# 3. Step-by-Step Operational Procedure

Step 1. Notify & Coordinate: Contact Control Room Operator (CRO) and confirm standby pump P-101B is online, primed, and taking the full amine circulation load without pressure spikes.

Step 2. Isolate Discharge: Slowly close discharge block valve XV-101A over a 2-minute duration to prevent water hammer on the column reflux line.

Step 3. Isolate Suction: Close suction block valve XV-101A-S firmly. Verify local pressure gauge shows 0 psig differential across suction line.

Step 4. Apply LOTO: Perform positive electrical lockout on P-101A motor breaker at MCC Substation 2. Apply multi-lock hasps and tags on both suction and discharge handwheels per STD-EI-002.

Step 5. Depressurize & Drain: Connect closed-loop drain hose to pump casing bleed valve. Slowly open bleed valve to drain residual amine into closed hazardous drain header. Verify zero residual liquid.

Step 6. Flush & Clean: Connect demineralized utility water flush to casing vent. Flush casing for 15 minutes until effluent is clear and pH tested neutral.

Step 7. Handover to Maintenance: Perform joint field walkdown with Maintenance Lead. Sign Permit to Work and authorize mechanical disconnection.

⚠ HIGH HAZARD WARNING Caution: Amine solution under pressure can release H2S pockets during initial bleed. Ensure respiratory protection or SCBA is immediately staged during initial casing bleed.